L20 Doors/ Shutters/ Hatches

610 Roller shutter doors

Specification Text

Supplier: 
HVP Security Shutters Ltd, 4 Grace Road West, Marsh Barton, Exeter, EX2 8PU. Tel: 01392 270 218. Fax: 01392 278 548. Email: info@flexiglide.com Website: www.flexiglide.com

Product reference: 
FlexiGlide Vision Air Sliding Folding Shutter

Arrangement: 
Horizontally operated top hung sliding folding shutter

Construction: 
Configuration A - Vertical, oblong slots: Constructed from 152mm wide x 1.8mm thick full height aluminium panels. The panels are punched by 6mm x 32mm vertical, oblong slots in evenly spaced columns and staggered rows. The panels are framed by full height butt hinges at 178mm centres.

Configuration B - Perforated holes: Alternatively, the panels can be perforated by 5mm diameter holes in a staggered pattern and fitted with 102mm high solid aluminium plates at the top and bottom of the closure. In this configuration the panels have a thickness of 3.2mm, and have solid aluminium plates at the top and bottom.

Track: 
The overhead track is 34mm wide x 46mm high and takes 29mm diameter roller trolleys. The rollers bear on a 7mm thick aluminium surface within the track. Track curves, where indicated, have 254mm or 356mm radii. A custom radius track is available as an option.

Finish as delivered: 
Standard factory finishes are 10 micron clear anodizing, K1285 (white), K70632 (light bronze), K71232 (medium bronze) or K90421 (black) Duracron acrylic paint. The complete shutter can be powder coated to most RAL colours.

Operation: 
Manual operation.

Locking Options: 
The grill incorporates 62mm x 33mm aluminium rectangular locking posts that are placed at regular intervals along the length of the curtain. Posts with floor bolts include dust free, stainless-top floor sockets. Lead and trailing end posts are cylinder controlled lock stiles with self-adjusting, cold rolled steel floor bolts and top bolts through the top of the track into the support. Intermediate locking posts with cold rolled steel floor bolts are supplied for (up to) every 3048mm of curtain. Bi-part assemblies are supplied as required. One locking post has a hook-bolt deadlock activated by keyed or thumb-turn cylinders. A second intermediate locking post with a steel floor bolt includes a full height channel to accept the hook-bolt deadlock.

Weight and Stacking: 
Grille weight is expressed in kg/square metre of clear opening. Vision Air - 10.7 kg/square metre. Minimum stacking is 87.5mm/metre of opening plus 89mm per locking member. The structural support must be capable of carrying the weight of a fully stacked door at any point along its length.

Other requirements: